Use only High Quality Synthetic Oil for Air cooled two cycle engines (not Outboard, not Lawn Boy from Wal-Mart)

Examples: Castrol TTS, Bel Ray MC1

Regardless of what the bottle says mix at 40:1 for Breaking in, afterwards you can go to up to 50:1

For Castrol TTS use 1 bottle per 5 gallons.

Use only 93 octane or higher (Avgas 100 LL if available)

The reason for the synthetic oil is less carbon build up that will clog the decompression hole and make the rings stick over time.

1. Run at 2500 to 3000 rpm for 2 minutes

2. Run at 3500 to 4000 rpm for 2 minutes

3. Let idle for 1 minute

4. Run at 4000 rpm for 1 minute

5. Let idle for 1 minute

6. Run at 4500 rpm for 1 minute

7. Let idle for 1 minute

Stop engine and let cool off completely

Repeat above procedure for a total of 1 hr.

You can break-in during short flights, not longer than 10 minutes.

Do not use full throttle for longer than 30 seconds until the engine has 4 hrs run time and after that only use full throttle to launch and no longer than 5 minutes. Your engine will thank you with long life.