PPG 2 Test

80 % passes

1. As a Powered Paraglider Pilots you are

a) free to fly anywhere as long as you stay below 3000 feet

b) regulated by FAR 103

c) allowed to fly above 18000 feet if in unrestricted airspace

d) b and c above

e) none of the above

2. By law you

a) must take at least 2 days of lessons before flying solo in your PPG

b) must take at least 3 days of lessons before flying solo

c) must take enough lessons until proficient before flying solo

d)  have to take a test before flying solo

e) none of the above

3. In order to legally take a passenger on a tandem flight you

a) need a tandem paraglider and tandem paramotor

b) must be certified as an advanced pilot

c) a and b above

d) need an exemption from the FAA

e) none of the above

4. Launching into the wind is

a) a good idea because it helps to inflate the Glider

b) not necessary if you have good skills and plenty of thrust

c) a bad idea because there will be too much drag

d) a good idea because it lowers your ground speed

e) a and d above

5. It's Saturday afternoon at 2 pm. You finally have the weekend off and are ready to fly.

It has been a beautiful, sunny day and the wind is light and variable. You can expect:

a) very smooth air

b) very turbulent air

c) it's hard to say until you get in the air

d) some amount of turbulence depending on the thermal activity that day

e) flying at 2 pm on a sunny day is most likely beyond the skill level of a PPG 2 rated pilot

f) d and e above

6. Same sceneario as above but the wind is gusting from 0 to 15 mph and shifting directions frequently

You can expect:

a) reasonably smooth air

b) to have a good flight as long as you stay upwind of your landing area

c) moderate to strong thermal induced turbulence

d) it's hard to say until you get airborne.

7. It's 7 pm on a summer afternoon. The winds have been light all day. You arrive at your launch area to find a steady 5 mph wind. You can expect:

a) smooth air and a nice flight

b) turbulent air due to too much wind

c) the winds to pick up

d) b and c above

8. It's 7 am just after daybreak. There is no wind. The sky is clear. You can expect:

a) a difficult launch

b) very smooth air

c) turbulent air

d) a and b above

e) none of the abov

9. The weather foecast calls for a cold front moving in and winds picking up this afternoon from the Northwest. You are ready to take off. Despite the forecast the winds are light and steady out of the Southwest and condtions seem very good.

a) The forecasters are always wrong, I'll go ahead and enjoy my flight

b) After I launch I watch for signs of the front and land befor it hits

c) If the front comes in while I am in the air I can expect very strong turbulence.

d) As the winds are picking up I may not be able to penetrate back to my launch

e) c and d above

f)  b and d above

10. The winds on the ground are  blowing at 15 mph. Your  Glider has a maximum speed of 20 mph

you

a)  have a minimum ground speed of 5 mph while flying into the wind

b) may not be able to penetrate into the wind once at higher altitude

c) need more skills for launching

d) b and c above

e) none of the above

11. Your Glider is rated DHV 2 and AFNOR Performance

a) That's the corresponding Glider for a PPG 2

b) you need more skills and experience when flying this Glider

c) ratings are not a concern in the US

d) this Glider is in most cases not suitable for a PPG 2

e) b and d above

12. Kiting the Glider on the ground is

a) a trick used in Europe

b) not necessary for a PPG pilot

c) recommended to practice often

d) giving you a good feel for the Glider

e) c and d above

13. You had two lessons and some groundhandling (kiting) excercise. You

a) are ready to fly solo and take home your equipment

b) most likely need more lessons

c) are ready to fly solo but must stay below 300 feet

d) none of the above

14. You live downtown in a large city and would like to fly over your appartment to take a picture.

a) Make sure to safety your camera

b) if you run out of gas and have to land you might get arrested

c) That's illegal because you are flying over a "congested area"

d) This would be an unsafe  practice

e) It's o.k. as long as you stay out of restricted airspace

f) b, c and d above

15. Your launch area is clear for 500 feet and the wind is blowing straight down the field. At the end of the

runway there are some 80 foot tall trees. You have launched and are coming up on the trees just below the

tops. You should

a) increase power so you can climb over the trees

b) expect rotor and sinking air and make a turn away from the trees

c) cut the power and make a sharp 180 degree turn to  avoid the trees

d) aim for a large branch and hold on when you hit

e) find a better field

f) b and e above

16. You are flying in strong conditions with lift and sink. The sink is as high as 1000 fpm.

a) You should be able to climb through it with full power

b) You should look for easier flying conditions as a PPG 2

c) You may not be able to climb while in the sink

d) As long as you stay above 1000 feet there is no problem

e) a and d above

f) b and c above

17. You live in Florida at Sea Level and have travelled with your PPG to Denver, CO (6000 feet) .You

a) may need to adapt to the higher altitude

b) may need to tune your motor for different density altitude

c) may have less thrust available

d) may have to run faster to get launched

e) will have higher airspeeds

f) all of the above

18. Big Ears are:

a) a problem with certain helmets

b) achieved by pulling on the "A" risers

c) a maneuver to increase the sink rate on your Glider

d) sometimes difficult to perform with high attachment points

e) c and d above

19. Pulling a lot of brake while under full power is

a) dangerous and may accidentally stall the Glider

b) good for increased pressure in the Glider

c) o.k. if only used intermittently (pumping) and helps with thermalling

d) b and c above

20. Your Propller turns to the left while looking at the prop from behind the motor. You should

a) favor left hand turns

b) make only hard left turns under power

c) favor right turns

d) keep an open path to the right of the launch area

e) use a lot of brake for left turns

f) c and d above

21. You are launching and have just lifted off. If you pick up your feet right away, you

a) will improve climb rate due to less drag

b) will climb slower due to heavier loading

c) may touch back down on your butt, possibly braking the propeller in the process

d) none of the above

22. You are aborting your launch because the Glider did not come up straight. You should

a) stop and wait until the Glider falls down

b) move backwards towards the Glider

c) move forward or sideways to avoid the lines and fabric touching the motor/Propeller

d) kill the engine

e) c and d above